SECTION 40 80 00

Universal Device Configuration Tablet PC

- PART 1	 GENERAL

- 1.01 SUMMARY

- Tablet PC device configurator which is suitable for both hazardous and non-hazardous areas for configuration and maintenance of field instruments.  The tablet PC supports the following device communications: HART, Wireless HART, Profibus DP/PA, Foundation Fieldbus, Modbus over serial and other specific vendor service protocols. The tablet PC is complete with pre-installed libraries, application software and is touch screen enabled to manage field instruments during their entire life cycle.

1.02 SUBMITTALS

- Innovations Brochure

- Technical Information

- Operating Instruction

- Manufacturer Information

PART 2 	PRODUCTS

2.01 MANUFACTURER

- One of the following:

- Endress+Hauser- Field Xpert SMT70

2.02 PRODUCT

A.	Universal device configuration supporting protocols such as HART,Wireless HART, Profibus DP/PA , Foundation Fieldbus, Modbus over serial and other specific vendor service comms using USB, Ethernet and WiFi connections.

- Touch-enabled, intuitive device configuration software in 20 languages.

- NAMUR NE 107 instrument diagnosis.

- Pre-Installed device driver libraries

- Fast connection to devices with a single click

- Automatic hardware detection

- Integrated Heartbeat Verification including PDF documentation

- High-performance robust table with 11.6” display and 14 hour battery.

- 5 MP camera on the back with auto focus and LED flash with 1500 mcd and 2 MP camera on the front.

- 1D/@D barcode scanner

- CPU - Intel Core i5-5200u processor (2.2 to 2.7 GHz)

- Memory 8 GB with DDR3L adapter

- Expansion slots – Micros SD card slot support for SDHC/SDXC and SIM card slot for WWAN data communication

- Storage – 256 B SSD M2-SATA in robust version

- Audio – Speaker 70 dB@ 0.5m Microphone

- Replaceable and rechargeable lithium polymer battery

- 11.6” LCD display with Gorilla Glass protector

- Resolution is 1366 x 768 pixels

- Communication interface by 2 x USB 3.0, Intel wireless, Bluebooth 4.0 and 46LTE WWAN + 12 channel GPS (optional)

- Windows 10 Pro (64 Bit) operating system

- IP65 rated degree of protection

- Hazardous area approval – NEC/CEC Class 1, Div.2, Group A-D T4.

- PART 	3	EXECUTION

- 3.01 EXAMINATION

- Ensure proper functioning of tablet PC and communication devices.

- Check proper validation of tablet tools system firmware, software licenses, patches, updates, user permissions and internet settings are correct.

- Enter proper user training of using this tool per job task procedures.