SECTION 40 80 00

Universal Device Configuration Tablet PC

- PART 1	 GENERAL

- 1.01 SUMMARY

- Field communicator tool, rugged tablet PC solution which is suitable for non-hazardous areas for configuration and maintenance of field instruments.  The tablet PC unit and universal device configuration management software supports the following device communications: HART, Wireless HART, Profibus DP/PA, Foundation Fieldbus, Modbus over serial and other specific vendor service protocols. The tablet PC solution is complete with pre-installed libraries, application software and is touch screen enabled to manage field instruments during their entire life cycle.

1.02 SUBMITTALS

- Innovations Brochure

- Technical Information

- Operating Instruction

- Manufacturer Information

PART 2 	PRODUCTS

2.01 MANUFACTURER

- One of the following:

- Endress+Hauser- Field Xpert SMT50

2.02 PRODUCT

A.	Universal device configuration supporting protocols such as HART,Wireless HART, Profibus DP/PA , Foundation Fieldbus, Modbus over serial and other specific vendor service comms using USB, Ethernet and WiFi connections.

- Touch-enabled, intuitive device configuration software in 20 languages.

- NAMUR NE 107 instrument diagnosis.

- Pre-Installed device driver libraries

- Fast connection to devices with a single click

- Automatic hardware detection

- Integrated Heartbeat Verification including PDF documentation

- High-performance robust tablet with 12.2” display and 8 hour battery.

- 5 MP camera on the back and 2 MP camera on the front.

- CPU - Intel® Sky Lake Core™ m3-7Y30

- Memory 4GB RAM / 128 GB SSD

- Interface – 3 USB, 1 HDMI, Serial port RS-232, RS485

- Communication - Ethernet RJ-45, Bluetooth 4.2, WiFi, WWAN LTE/ GPS

- Storage – 128 GB SSD up to 512GB

- Audio – 2x speaker 0.8 Watt + 3.5mm audio out

- Hot Swap, Replaceable and rechargeable lithium polymer battery

- Touchscreen – 10 point capacitive with Gorilla Glass 3 protection

- Resolution is 1920 x 1200 pixels

- Windows 10 IoT operating system

- Protection class - IP65 rated degree of protection, MIL STD-810G

- Operating temperature -10 to 50 deg Celsius

- PART 	3	EXECUTION

- 3.01 EXAMINATION

- Ensure proper functioning of tablet PC and communication devices.

- Check proper validation of tablet tools system firmware, software licenses, patches, updates, user permissions and internet settings are correct.

- Enter proper user training of using this tool per job task procedures.